CODE FOR RENEWABLE GRASS CUTTING MACHINE

/*

Project: Renewable Solar Grass Cutter Machine

Controller: Arduino Uno

Motor driver: L298N

Note: Solar panel charges the battery;

the battery powers the Arduino and motors.

*/

// Blade motor driver pins

const int bladeIN1 = 8;

const int bladeIN2 = 9;

const int bladeEN = 10;

// Optional movement motor driver pins

const int moveIN1 = 4;

const int moveIN2 = 5;

const int moveEN = 6;

// Start/stop switch

const int startButton = 2;

// Optional obstacle sensor

const int obstaclePin = 7;

bool machineRunning = false;

void setup() {

pinMode(bladeIN1, OUTPUT);

pinMode(bladeIN2, OUTPUT);

pinMode(bladeEN, OUTPUT);

pinMode(moveIN1, OUTPUT);

pinMode(moveIN2, OUTPUT);

pinMode(moveEN, OUTPUT);

pinMode(startButton, INPUT_PULLUP);

pinMode(obstaclePin, INPUT);

stopMachine();

Serial.begin(9600);

}

void loop() {

// Press the button to toggle the machine

if (digitalRead(startButton) == LOW) {

delay(40); // Basic debounce

if (digitalRead(startButton) == LOW) {

machineRunning = !machineRunning;

while (digitalRead(startButton) == LOW) {

delay(10);

}

}

}

// Stop if an obstacle is detected

// Assumes the sensor output becomes HIGH

if (machineRunning && digitalRead(obstaclePin) == HIGH) {

stopMachine();

machineRunning = false;

Serial.println("Obstacle detected. Machine stopped.");

}

if (machineRunning) {

// Run blade motor

digitalWrite(bladeIN1, HIGH);

digitalWrite(bladeIN2, LOW);

analogWrite(bladeEN, 200);

// Move forward

digitalWrite(moveIN1, HIGH);

digitalWrite(moveIN2, LOW);

analogWrite(moveEN, 150);

} else {

stopMachine();

}

}

void stopMachine() {

digitalWrite(bladeIN1, LOW);

digitalWrite(bladeIN2, LOW);

analogWrite(bladeEN, 0);

digitalWrite(moveIN1, LOW);

digitalWrite(moveIN2, LOW);

analogWrite(moveEN, 0);

}